# Research Pipeline

## Name

ahtm_reflex_cam

## Title

Hardware-accelerated reflex memory for accelerating first-order sequence inference in hierarchical temporal memory on IoT streams

## Short Hypothesis

Integrating a spinal-cord-inspired reflex memory block with a hardware CAM lookup into HTM will reduce median reaction latency for first-order repetitive sequences by at least 50% and energy per lookup by at least 30% compared to vanilla HTM, while keeping sequence prediction F1 score within 2 percentage points of baseline.

## Related Work

Existing methods implement Hierarchical Temporal Memory (HTM) for unsupervised sequence learning and use software or hashed associative stores for sequence memory. Prior hardware approaches accelerate associative lookup using LUT-based CAMs (fast, low capacity) or BRAM/URAM hashed stores (slower, larger capacity). Two-timescale plasticity (fast/slow learning) has been proposed to balance rapid adaptation and long-term memory. Limitations: (1) HTM sequence memory is bottlenecked by repeated, first-order predictions in highly repetitive data, causing unnecessary TM traversal and energy use; (2) hashed fallback introduces collision risk and verification overhead; (3) prior hardware prototypes trade off latency for capacity without a unified reflex cache design; (4) reproducibility and fixed-point register encodings are often under-specified, limiting comparison. No prior design combines a reflex cache (for fast first-order matches), explicit consolidation (fast→slow transfer), and a hierarchical CAM/BRAM backend with a pre-registered experimental verification plan.

## Abstract

We address latency and energy bottlenecks in unsupervised sequence learning for repetitive Internet-of-Things (IoT) streams by introducing a reflex memory block into Hierarchical Temporal Memory (HTM). The reflex block stores frequently occurring first-order transitions and is implemented as a hardware-accelerated associative cache using a low-latency CAM for small sets and a hashed BRAM fallback for larger capacity. Fast plasticity updates in the reflex cache enable rapid adaptation; periodic consolidation transfers learned reflexes into the slower HTM substrate to preserve multiorder sequence inference. Main contributions are: (1) a compact reflex memory design and MMIO mapping for hardware integration, (2) a two-timescale learning protocol (fast update + synaptic tagging and replay-based consolidation), and (3) a reproducible experimental protocol with hardware-in-the-loop measurement. Expected outcome: substantially lower decision latency and energy on repetitive first-order sequences while maintaining HTM’s multiorder prediction performance and providing quantifiable safety and collision bounds for hashed fallbacks.

## Experiments

Datasets: (1) synthetic_k_order_markov: deterministic Markov generators with k ∈ {1,2,3,4}, sequence length 1e6 timesteps, urgent token rate {0.01,0.05}, noise bitflip rates p_flip ∈ {0,0.01,0.05,0.1}, seeds s=0..29 (30 independent runs); (2) real_iot_sensor_streams: 10 traces of sampled sensor telemetry, each 5e5 timesteps, seeds for preprocessing s=0..9; (3) robot_control_streams: closed-loop sensorimotor traces, 2 tasks × 1e5 timesteps. Baselines: vanilla TM (no reflex), AHTM without consolidation (reflex fast-only), AHTM with hashed-only store (no CAM verification), randomized-key control. Hardware setup: FPGA HIL on ZCU104 (or equivalent), frequency 250 MHz, LUT-CAM backend (configurable entries up to 8k) and BRAM hashed backend; fixed-point Q1.15 registers for hyperparameters. Evaluation metrics: median and 95th-percentile reaction latency (ms), reflex precision/recall, sequence prediction F1, energy per lookup (pJ) measured via instrumented power traces, consolidation throughput (entries/s), and forgetting curves (retention vs time). Experimental protocol: run each condition N=30 seeds, sweep α_fast ∈ {0.05,0.1,0.2,0.4} and τ_e ∈ {25,50,100,200} (eligibility trace timesteps), and hysteresis n ∈ {1,2,3}; perform ablation comparing consolidation ON vs OFF and CAM vs hashed backend; record per-run telemetry. Statistical tests: two-way ANOVA for α_fast×τ_e effects on latency and F1, post-hoc Bonferroni corrections, report 95% confidence intervals computed by bootstrap over seeds. Reproducibility artifacts: provide fixed seed lists, dataset generator code, MMIO register map, and cycle-accurate simulator.

## Risk Factors And Limitations

Technical failure modes: hashed-tag collisions causing false reflex triggers (false positives) and integrity violations in verification-limited fallbacks; runaway updates if α_fast and eligibility traces are poorly quantized, especially under fixed-point rounding. Hardware constraints: FPGA LUT budget may limit CAM entries (N << 8k), forcing frequent fallback to higher-latency BRAM and degrading latency gains. Measurement limits: power/energy estimates depend on instrumentation resolution and may vary across boards; small sample sizes can inflate CI. Data and evaluation biases: synthetic Markov tasks emphasize repetitiveness and may overstate benefits versus diverse real-world streams; closed-loop rollback (inhibit) latency can destabilize downstream controllers in time-critical systems. Scalability and generalization: reflex cache benefits concentrate on first-order, repetitive patterns and may not improve or can harm multiorder learning if consolidation bandwidth is insufficient. Reproducibility risks: inconsistent fixed-point encodings or MMIO bitfield mappings across implementations will change numeric behavior and complicate comparisons.